ChestXsim – Command Line Cheatsheet

This document provides instructions to run the command-line tools included in ChestXsim package, either:

- Locally, after installing the package on your machine, or

- Inside a Docker container, by building and running the provided Docker file.

The main CLI tools are:

- run_simulation – run DCT simulations form chest CT volumes

- interpolate – resample CT volumes to match DCT reconstructed resolution.

Regardless of whether you use local or Docker execution, you must first clone the repository and work from the project root:

git clone https://github.com/mabellag/ChestXSim.git

cd ChestXSim

The project root is the directory that contains:

- pyproject.toml

- scr/chestxsim/

- inputs/

- materials/

- settings/

- results/

- examples/

All the relative paths in the examples below assume that commands are executed from this location.

- Local CLI Usage (Windows, conda envirionment)

  - Requirements

- Windows 10/11

- An NVIDIA GPU with NVIDIA CUDA Toolkit 12.x

- NVIDIA drivers compatible with CUDA 12.x.

- A CUDA-enabled PyTorch build

  - Environment and package installation

Create and activate conda environment

conda create -n chestxsim python=3.10

conda activate chestxsim

Install core dependencies inside the environment:

- A CUDA-enabled PyTorch build

pip install torch==2.1.0 torchvision==0.16.0 --index-url https://download.pytorch.org/whl/cu121

- CuPy compiled for CUDA 12.x

pip install cupy-cuda12x

- ASTRA Toolbox installed via conda on Windows

conda install conda-forge::astra-toolbox

Install ChestXsim package from the project root:

pip install -e .

Verify installation:

python -c "import chestxsim; print('ChestXsim successfully installed')"

python -c "import torch; print('PyTorch CUDA:', torch.cuda.is_available())"

  - Local run_simulation usage

General syntax

run_simulation --input PATH_TO_CT_STUDIES --config settings/CONFIG.json [--mode 0|1|2] [--output OUTPUT_DIR]

Arguments

--input / -i : Folder with CT data (DICOM, RAW, or ChestXsim outputs)

--config / -c : JSON configuration file defining geometry and pipeline

--mode / -m : 0 = preprocessing only, 1 = projection only, 2 = reconstruction only if  omitted → full pipeline (preprocessing + projection + reconstruction)

--output / -o : Base output directory for all saved results, if omitted, the default results folder is used.

Full simulation examples

(Results saved to default result folder)

Monochromatic projections + FDK reconstruction:

run_simulation -i ./inputs -c ./settings/volumeRAD_mono_FDK.json

Polychromatic projections + FDK reconstruction:

run_simulation -i .\inputs -c .\settings\volumeRAD_poly_FDK.json

Geometry-only projections + SART reconstruction:

run_simulation -i .\inputs -c .\settings\volumeRAD_geom_SART.json

Running modules independently

Use the –output option to keep experiments organized and to maintain full control when running modules independently. This is especially useful when:

- You want to avoid re-running preprocessing and instead reuse the same preprocessed volume to simulate multiple projection scenarios (e.g., changing the number of projections, tube voltage, noise settings, etc.).

- You want to test different reconstruction techniques or parameters starting from the same set of saved projections.

In this example, we run a polychromatic simulation in three stages, all sharing the same experiment root folder: .\results\volumeRAD_poly_FDK.

1) Preprocessing only

Generates bed-removed, segmented and converted CT volumes under the chosen output folder.

run_simulation -i .\inputs -c .\settings\volumeRAD_poly_FDK.json -o .\results\volumeRAD_poly_FDK -m 0

2) Projection only

Reuses the preprocessed density volumes as input to generate projections.

run_simulation -i .\results\volumeRAD_poly_FDK\CT_converted\density -c .\settings\volumeRAD_poly_FDK.json -o .\results\volumeRAD_poly_FDK -m 1

3) Reconstruction only

Reconstructs from the previously saved projections.

run_simulation -i .\results\volumeRAD_poly_FDK\EnergyProjections -c .\settings\volumeRAD_poly_FDK.json -o .\results\volumeRAD_poly_FDK -m 2

  - Local interpolate usage

General syntax

interpolate --input PATH_TO_CT_STUDIES --vx_xyz vx_x,vx_y,vx_z [--output OUTPUT_DIR]

Arguments

--input / -i: Folder containing CT volumes after bed removal  to resample.

--vx_xyz / -vx: Target voxel size in mm, given as vx_x,vx_y,vx_z. By default, it uses  1.25,5.0,1.25 .

--output / -o : Output directory to save results.

Example: resample bed-removed CT to DCT voxel size (1.25 × 5.0 × 1.25 mm)

interpolate -i .\results\volumeRAD_poly_FDK\CT_without_bed -vx 1.25,5.0,1.25 -o .\results\volumeRAD_poly_FDK

- Docker-based CLI Usage (Any OS)

2.1. Requirements

The simplest way to use ChestXsim via Docker is to install Docker Desktop.

- Download and install from https://docs.docker.com/desktop/

- On Windows, ensure that WSL2 is installed or enabled during setup

To be able to use  GPU acceleration inside Docker the host system must have:

- An NVIDIA GPU.

- NVIDIA drivers compatible with CUDA 12.x.

- The NVIDIA Container Toolkit. Windows users do not need to install this manually - Docker Desktop + WSL2 provide GPU passthrough automatically. Linux users must install this manually: https://docs.nvidia.com/datacenter/cloud-native/container-toolkit/latest/install-guide.html

  - Build the ChestXsim Docker image

After cloning the repository, navigate to the project root and build the image:

docker build -t chestxsim .

  - Required volume mounts and environment variables

Inside the container, ChestXsim needs access to the project structure (for inputs, configuration files, default result paths, etc.)

- Project root bind mount: -v <PROJECT_ROOT>:/app

- Environment variable: -e CHESTXSIM_ROOT=/app

where <PROJECT_ROOT> is the path to the project root location.

The general pattern for executing any CLI tool:

docker run --rm --gpus all -v <PROJECT_ROOT>:/app -e CHESTXSIM_ROOT=/app chestxsim <cli-command> [arguments]

  - run_simulation via Docker

Full simulation examples

docker run --rm --gpus all -v <PROJECT_ROOT>:/app -e CHESTXSIM_ROOT=/app chestxsim run_simulation -i /app/inputs -c /app/settings/<CONFIG>.json -o /app/results/volumeRAD_poly

Where <CONFIG>.json is one of:

- volumeRAD_mono_FDK.json

- volumeRAD_poly_FDK.json

- volumeRAD_geom_SART.json

Running modules independently

docker run --rm --gpus all -v <PROJECT_ROOT>:/app -e CHESTXSIM_ROOT=/app chestxsim run_simulation -i /app/<INPUT FOLDER> -c /app/settings/volumeRAD_poly_FDK.json -o /app/results/volumeRAD_poly_FDK -m <MODE>

For the volumeRAD_poly_FDK example:

- Preprocessing: <INPUT_FOLDER> = inputs, <MODE> = 0

- Projection: <INPUT_FOLDER> = outputs/volumeRAD_poly_FDK/CT_converted/density, <MODE> = 1

- Reconstruction: <INPUT_FOLDER> = outputs/volumeRAD_poly_FDK/EnergyProjections, <MODE> = 2

2.5. interpolate via Docker

docker run --rm --gpus all -v <PROJECT_ROOT>:/app -e CHESTXSIM_ROOT=/app chestxsim interpolate -i /app/results/volumeRAD_poly_FDK \CT_without_bed -vx 1.25,5.0,1.25  -o /app/results/volumeRAD_poly_FDK